QP Code: MNM201

M. Sc Nuclear Medicine Technology 2nd Year

(BF/2022/04)

Nuclear Medicine imaging and Counting Instrumentation

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

Write about the following:

1.	Explain paraysable and non paralysable system.				[5]

2.	What is quenching? Explain its role in Geiger Muller counter.		[5]

3.	What are the different methods of background subtraction used in nuclear medicine?										[5]

4.	What is energy resolution and efficiency of a detector?			[5]

5.	Describe working principles of thyroid uptake probe.			[5]

6.	Describe daily quality control tests performed for SPECT-CT system.	[5]

7.	Quality control of dose calibrator.						[5]

8.	Low energy high resolution collimator.					[5]

Section-B				Marks= 40

Write about the following:

1.	What is integral and differential counting?					[5]

2.	Compare Nal(TI) scintillator detector with semiconductor detector.		[5]

3.	What is a role of preamplifier and amplifier in nuclear medicine instruments?												[5]

4.	What is COR? Explain its significance.					[5]

5.	Discuss working principles of SPECT-CT system.				[5]

6.	Write a short note on photomultiplier tube.					[5]

7.	What is dead time and recovery time?					[5]

8.	Gas field detectors.								[5]

--------------------

QP Code: MNM202

M. Sc Nuclear Medicine Technology 2nd Year

(BF/2022/04)

Principles and Practical of Radiological Protection, Radiation Dosimetry, Radiobiology and Hospital Practice

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

Write about the following:

1.	Discuss the units for absorbed dose and exposure.					[5]

2.	Discuss the general principles of radiation protection.				[5]

3.	How the radioactive decontamination of labs, clothes and hand is done?		[5]

4.	Elaborate on the role of ICRP and IAEA.						[5]

5.	What are the natural sources of radiation exposure? Also discuss the exposure limits prescribed by AERB.									[5]

6.	What is the use of survey meters and dose calibrators in radiation facility setup?	[5]

7.	What are the various forms of solid radioactive waste in nuclear medicine setup and the basic ways of disposal?						 		[5]

8.	Discuss the licensing procedures under atomic energy rules.				[5]

Section-B				Marks= 40

Write about the following:

1.	Classification of radioactive material for transportation.				[5]

2.	Derive MIRD equation.								[5]

3.	In- utero effects of radiation.								[5]

4.	Design and safety aspects of planning a nuclear medicine department.		[5]

5.	Emergency plans for lost source and stuck source.					[5]

6.	How the preparation of patients for general nuclear medicine procedures dose?	[5]

7.	What is neutron activation analysis?							[5]

8.	What is the distance required to reduce the radiation level from a 20MBQ 60Co source to 0.025 mGy/hr? (Exposure rate constant for 60Co-3.1 mGy/h/MBq at 1cm).		[5]

--------------------

QP Code: MNM203

M. Sc Nuclear Medicine Technology 2nd Year

(BF/2022/10)

Principles and Practical of Radio pharmacy and Radionuclide Generators, SPECT and PET Radiopharmaceuticals

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

Write about the following:

1.	What are buffer solutions? Explain in detail.						[5]

2.	Define Metastable and Isomeric transition.						[5]

3.	What account for nuclide stability and instability?					[5]

4.	Define Thermal Neutron. Elaborate in detail.				 		[5]

5.	Define Bronsted Acid and Base. Give examples.					[5]

6.	Standard Operating Procedure of laboratory safety to perform organic reactions.	[5]

7.	What do Heterocylic compounds stand for? Explain with examples.		[5]

8.	Define catalyst with example, its role in chemical reactions.				[5]

Section-B				Marks= 40

Write about the following:

1.	In the hot lab which cold kit requires heating step for the preparation of the radiopharmaceutical? Elaborate in detail?						[5]

2.	What are Auger electrons and their role in radionuclide therapy?		 	[5]

3.	Define mode of radioactive decay in context to Beta and Positron decay with one example each.										[5]

4.	Classified Nuclear Reaction and its utility in Nuclear Medicine?			[5]

5.	What does Compton Effect stand for? Explain the annihilation radiation.		[5]

6.	Explain Nucleophilic substitution and Electrophilic substitution organic reactions? Explain with examples.							 	[5]

7.	Define role of co-ordination chemistry with examples of various chelating agents used in Nuclear Medicine.									[5]

8.	What are macromolecules and their importance in radiopharmaceuticals development?											[5]

--------------------

QP Code: MNM204

M. Sc Nuclear Medicine Technology 2nd Year

(BF/2022/04)

Nuclear Medicine imaging and In-vivo Counting PET Imaging Cyclotron and Allied Instrumentation

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

Write about the following:

1.	What are the methods of doing diuretic renogram?					[5]

2.	Describe various radiopharmaceuticals used for skeletal imaging.			[5]

3.	Describe the steps of performing denatured RBC studies following spleenectomy.	[5]

4.	What is perchlorate discharge test and how is it performed?				[5]

5.	Describe how to perform gall bladder ejection fraction studies.			[5]

6.	Briefly describe 99mTc	-DTPA aerosol studies for mucociliary clearance.		[5]

7.	Why one needs to do stress MPI if Rest myocardial studies was normal?		[5]

8.	Preparation of patient for 131I-MIBG therapy.					[5]

Section-B				Marks= 40

Write about the following:

1.	Describe various types of medical cyclotrons.					[5]

2.	Non cyclotron produced PET radiopharmaceuticals currently used in India.	[5]

3.	What is CTDI?								 	[5]

4.	What is OSEM reconstruction technique in PET imaging?				[5]

5.	Define precision, accuracy & sensitivity of the TSH assay.				[5]

6.	How nuclear medicine is helping in surgical epilepsy cases?	 		[5]

7.	What are the new techniques of detection of anti-TPO antibody estimation?	[5]

8.	Describe Russel’s two-samples methods of GFR estimation?			[5]

--------------------