QP Code: NMS201

M. Sc Nuclear Medicine Technology 2nd Semester

(BF/2023/10)

Radiation Detection and Measurement

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

1.	Write about gas filled detectors. Different types of detectors. Write in detail.	[10]

OR

What are semi-conductor detectors? Different types of semiconductor detectors and principles of detection?

2.	Write about contamination monitoring instruments used in nuclear medicine.	[5]

3.	What are the basic requirements for calibration and various parameters checked during calibration?											[5]

4.	What is dual radionuclide counting? Write in detail.					[5]

5.	Write about sodium iodide detector in detail.						[5]

6.	What is Compton edge and Compton valley? Write in detail.			[5]

7.	Write about statistical aspects of gamma ray and beta ray counting.			[5]

Section-B				Marks= 40

1.	Write in detail about basic design of gamma camera and its functions.		[10]

OR

Write about OSLD reader and its role in medical and research applications.

2.	Principles of whole body counting and design of whole body counting system?	[5]

3.	What is REM counter? Write in detail.						[5]

4.	Write about uncertainty calculation and error propagation?				[5]

5.	Write about components in liquid scintillation detection?				[5]

6.	What is Energy resolution and calibration? Write in detail.				[5]

7.	What is geometric efficiency? Write in detail.					[5]

---------------------

QP Code: NMS202

M. Sc Nuclear Medicine Technology 2nd Semester

(BF/2023/10)

Basic concepts of Electronics Biomedical Instruments and Techniques

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

1.	What is an electrocardiograph? Explain its principle of operation and the different components of an ECG machine.							[10]

OR

What are pnp and npn transistors? How do they differ in their properties and working?

2.	Describe the different types of ionization techniques used in mass spectrometry.	[5]

3.	What is the basic principle of atomic absorption spectroscopy and hoe does it work to detect analytes in biological samples?							[5]

4.	How are the concepts of chemical shift, T1, and T2 relaxation times used in magnetic resonance imaging (MRI)?									[5]

5.	Explain the concept of the crossover effect in X-ray imaging. How does it affect the contrast and sharpness of the image?							[5]

6.	What is ultrasound? Describe its basic principle of operation.			[5]

7.	What are the physical properties of ultrasound waves?				[5]

Section-B				Marks= 40

1.	What is laser-tissue interaction? Explain the different mechanisms of laser-tissue interaction?										[10]

OR

Explain the working principles of a photmultiplier (PM) tube and its application in electronics Draw diagram of PMT.							[10]

2.	What are intrinsic and extrinsic semiconductors? Explain their differences.		[5]

3.	What are the basic logic gates? Explain their truth tables and their logic symbols.	[5]

4.	What is a thermoresistor? How does it differ from a thermistor? Give an example of an application.										[5]

5.	Discuss the concept of coincidence and anticoincidence circuits. Explain their applications in nuclear electronics.							[5]

6.	What are the physical properties of ultrasound waves?				[5]

7.	What is a time-to-pulse-height-converter (TPHC) and how does it work.		[5]

---------------------

QP Code: NMS203

M. Sc Nuclear Medicine Technology 2nd Semester

(BF/2023/10)

Applied Mathematics, Biostatistics and Computer Applications

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

1.	Let A= and B=. Calculate A+B, A-B AB and BA. 		[10]

OR

Calculatess inverse and adjoint matrices of A and B

2.	Define simple regression.								[5]

3.	Explain the difference between a sample and a population?				[5]

4.	Draw an Ogive of the following data?						[5]

| Height of groups (cm) | 150-152 | 152-154 | 154-156 | 156-158 | 158-160 | 160-162 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of groups | 10 | 15 | 17 | 19 | 24 | 30 | 115 |

5.	What is a “Test of significance”? Name any two tests of significance.		[5]

6.	Integrate 1/ x+xlogx									[5]

7.	Explain the function of CPU in a computer?						[5]

Section-B				Marks= 40

1.	If x+ y =, find values of X and Y.					[10]

OR

Integrate the following   and

2.	Write short note on normal distribution.						[5]

3.	Explain briefly different types of correlation.						[5]

4.	Write short note on computer memory.						[5]

5.	What is a chi square test and where is it used.					[5]

6.	Draw a scatter diagram of the following data?					[5]

| Counter per min | 12500 | 11100 | 9800 | 8700 | 6300 |
| --- | --- | --- | --- | --- | --- |
| Time points (minutes) | 30 | 60 | 90 | 120 | 150 |

7.	Evaluate the product of (3) * ( +7)					[5]

---------------------

QP Code: NMS204

M. Sc Nuclear Medicine Technology 2nd Semester

(BF/2023/10)

Radioisotope Application and Radiation Safety

###### M.M.: 80								      Time: 3 Hours

Note:	1.)	Attempt all questions.

2.)	ATTEMPT BOTH PARTS IN SINGLE ANSWER BOOK ONLY.

3.)	NO SUPPLEMENTARY SHEET SHALL BE ALLOWED/ PROVIDED

4.)	The Student must write Q.P. Code in the space provided on the Title page of the Answer Book.

Section-A				Marks= 40

1.	Discuss the biomedical uses/application of 3H and 32P 				[10]

OR

Elaborate on the disposal criteria with regard to effluent coming from isolation wards of patients undergone radioiodine therapy.

2.	Principle of radiation protection.							[5]

3.	Importance of post stress imaging of myocardium.					[5]

4.	Elaborate on the design, area and floor requirements for planning a radioisotope laboratory.										[5]

5.	Discuss thyroid suppression test.							[5]

6.	Decontamination procedure for hands and clothes					[5]

7.	Principle of immune radiometric assay (IRMA).					[5]

Section-B				Marks= 40

1.	Elaborate on internal radiation hazard and its control.				[10]

OR

Importance of standard, phantom, shielding and collimation for thyroid radioactive uptake measurements

2.	Radionale of targeted alpha therapy in Nuclear Medicine.				[5]

3.	Maximum permissible exposure limits for occupational worker and pregnant radiation worker.						 				[5]

4.	Procedure for radio labeling RBC’s with 51Cr.					[5]

5.	What is Urea breath test?								[5]

6.	Differentiate between Type A and Type B radioactive transport packages.		[5]

7.	Importance of splenic sequestration of 99m Tc RBCs.				[5]

---------------------